Monthly Updates for SDE Layers

7/16/2013

updated 12/27/2019 – TPS. New section on updating the PT geodatabase and new info in Test SDE Financing section.

Update 7/14/2022 – TPS. New updates to Water Rights: Long Term section in Test SDE

Update Overview:

Make sure gdbs in [z_GDBs] and g:\temp are current by checking the date of the geodatabase tables/feature classes in ArcCatalog (the “Modified” column needs to be showing on the Contents tab), then do this.

This updates SDE test server and pushes final to production server

(link to older doc)

The following updates should be performed at least once a month, preferably the first few days of the month.

Update the TEST SDE

Open ArcCatalog

Open the ArcToolbox

Load the Toolbox: [\\OWRBGIS\GeoDat\GIS\ArcGIS\Toolboxes\Z_SDE_Layers_Update108.tbx ]

Run the Following Models/Complete the following:

Dam Inventory:

- Update DI on Test (G:\Temp)

- Update DI on Production and CSA

- Delete Metadata Log

Financing: Important: Before you run the model, check the FA_Loans_Grants_NP table in the Financing.gdb. If there are any entries where the APPSTATUS is Approved, you need to update OWRBGIS.FA_Master_Entity_Points on OWRT, then copy it to OWRP, then rerun the tools in \\owrbgis\geodat\GIS\ArcGIS\Toolboxes\ZFinancingUpdate106.tbx to make sure the new loan/grant point appears.

1a) Update FA Loans Grants on Test

2a) Update FA Loans Grants on Prod and CSA

2b) FA_Metadata_Templates_Update.py

2c) Metadata Import

3) Update Master Entries to Prod (Only after Master Entity Points layer has been updated)

Water Quality:

1) Update WQ Layers on Test from G: - Do as needed, not monthly

Wells:

Important:  Look to see if the Well_Logs.gdb is present and up to date in both a:\z_GDBs and g:\temp.  If not then the nightly script failed and the toolbox needs to be run manually in ArcCatalog using the Z_Well_Logs_106 toolbox.  (Please note that the two “c” portions need to be switched in order.)  Please note that “p” will probably have to be run after hours when everyone is logged out of ArcGIS products.

1) Update Wells Test

2) Update Wells Production and CSA

3UpdateWLMetadataDate

4UpdateWLProdMetadata and CSA Metadata

Water Rights:

Important:  Look to see if the WaterRights.gdb is present and up to date in both a:\z_GDBs and g:\temp.  If not then the nightly script failed and the toolbox needs to be run manually in ArcCatalog using the Z_LT_Water_Rights_102 toolbox.  (Please note that the two “h” portions need to be switched in order.)  Please note that “m” and “n” will probably have to be run after hours when everyone is logged out of ArcGIS produtcts.

Long Term

Update LT Data

- Update WR Permit Statistics Table

- Update LT Lands Points Metadata

- Update Dashboard Date

PTs

Updating the Provisional Temporary Permits GIS Layers

Oil and Gas File Geodatabase – run every 6 months

Updating the Oil and Gas Water Rights FGDB

Check the updated layers on Test